Revised

November 1980

MATERIAL SAFETY DATA SHEET

(Approved by U.S. Department of Labor "Essentially Similar" to Form LSB-OOS-4)

NPVLA6-70

255-04

Section I

MANUFACTURER'S NAME

Hysan Corporation

STREET ADDRESS

919 West  38th Street

CITY, STATE. AND Z\P CODE

Chicago,   IL    60609

EMERGENCY TELEPHONE NO.       312/376-8900

CHEMICAL NAME AND SYNONYMS

N.A.

TRADE NAME

M-60, Glass Cleaner

CHEMICAL FAMILY

Glass Cleaner

FORMULA

N.A.

NAZARDOUS INGREDIENT^

PAINTS, PRESERVATIVES, & SOLVENTS

PIGMENTS

tlv (Units)

SOLVENTS

TLV (Unlti)

CATALYST

ADDITIVES

-IICLE

OTHERS

HAZARDOUS MIXTURES OF OTHER LIQUIDS, SOLIDS, OR GASES

tlv

(Units)

2-Propanol

C.A.S.  No.   67-63-0

400 ppm

2-Buoxyethanol

C.A.S. No.   111-76-2

25 ppm

Propellant:  c3-c4 Alkane Blend

C.A.S. No.       68475-59-2

800 ppm

Section III-PHYSICAL DATA

(ON  CONCENTRATE)"

BOILING POINT CF.)

IBP  200 F

SPECIFIC GRAVITY (H,O=1)

0.98

VAPOR PRESSURE (mm Hg.)

20 mm Hg  & 20 C

PERCENT  VOLATILE BY VOLUME (%)

100%

VAPOR DENSITY (AIR=1)

SOLUBILITY IN WATER

0.7

Complete

EVAPORATION  RATE

.

0.6

APPEARANCE AND ODOR

Clear thin liquid pleasantly scented.

Section IV - FIRE AND EXPLOSION HAZARD DATA

ASH POINT (METHOD USED)	o

EXTINGUISHING MEDIA

101 F

T.C.C.

Water, CO2

FLAMMABLE LIMITS

2

12

•SPECIAL FIRE FIGHTING PROCEDURES

Keep containers cool.  Use shielding to protect personnel against bursting, rupturing or venting containers.

UNUSUAL FIRE AND EXPLOSION HAZARDS	At elevated temperatures (over 130 F) containers may vent, rupture or burst.

Revised  November 198fi

255-04

SECTION V - Health Hazard Data

THRESHOLD LIMIT VALUE

>1000

ppm

EFFECTS_,OF OVEREKPOSURE	#

Eyes - May cause irritation upon prolonged contact.

EMERGENCY AND FIRST AID PROCEDURES

In case of contact with eyes, flush with water.

SECTION VI - Reactivity Data

STABILITY

UNSTABLE  X STABLE

CONDITIONS TO AVOID

u

Heat

INCOMPATABILITY NONE

HAZARDOUS DECOMPOSITION PRODUCTS

Oxides of carbon and nitrogen formed upon combustion

formed upon  combustion.

HAZARDOUS POLYMERIZATION

MAY OCCUR

WILL NOT OCCUR X

CONDITIONS TO AVOID

,,

SECTION VII - SPILL or LEAK PROCEDURES

•teps TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED        _	-

Remove sources of flame, spark or heat.

Provide ventilation.  Wipe up any spills.

WASTE DISPOSAL METHOD

Do not puncture or incinerate containers.  Give to a disposal service equipped to safely handle and dispose of pressurized containers.

| SECTION VIII – Special Protection Information |  |  |
| --- | --- | --- |
| RESPIRATORY PROTECTION (Specify type) None Required |  |  |
| VENTILATION | LOCAL EXHAUST None Required | SPECIAL rf N.A. |
|  | MECHANICAL (General) Maintain Adequate Ventilation | OTHER N.A. . |
| PROTECTIVE GLOVES EYE PRO! None Renuired |  | ECTION Safetv Glasses |
| OTHER PROTECTIVE EQUIPMENT None Required |  |  |
|  |  | - |

Section IX - SPECIAL PRECAUTIONS

PRECAUTIONS TO BE TAKEN  IN HANDLING  AND  STORING

Caution:Contents under pressure.  Do not store where temperatures mav exceed 120°F. Keep out of reach of children.  Avoid contact with eyes.  Read entire label before using.

OTHER PRECAUTIONS  READ ENTIRE LABEL BEFORE USING.